# Data Requirements for AI Wastewater Treatment Project

To develop robust Machine Learning models for aeration control, dosing optimization, and anomaly detection, the following historical time-series data is requested. A continuous period of 6 to 12 months is highly required.

| Category | Parameter | Description | Preferred Resolution | Importance for AI Models |
| --- | --- | --- | --- | --- |
| Water Quality (Influent) | pH | Acidity/Alkalinity of incoming water | Daily Lab Log | High |
| Water Quality (Influent) | BOD & COD | Biological & Chemical Oxygen Demand | Daily Lab Log | High (Load Prediction) |
| Water Quality (Influent) | TSS & TDS | Suspended & Dissolved Solids | Daily Lab Log | High |
| Water Quality (Influent) | O&G | Oil & Grease | Daily Lab Log | Medium |
| Water Quality (Influent) | Ammonia/Nitrogen | Nutrient loading | Daily Lab Log | High |
| Water Quality (Influent) | Temperature | Temperature of incoming water | Daily Lab Log | Medium |
| Water Quality (Effluent) | pH | Acidity/Alkalinity of outgoing water | Daily Lab Log | High (Compliance Target) |
| Water Quality (Effluent) | BOD & COD | Biological & Chemical Oxygen Demand | Daily Lab Log | High (Compliance Target) |
| Water Quality (Effluent) | TSS & TDS | Suspended & Dissolved Solids | Daily Lab Log | High (Compliance Target) |
| Water Quality (Effluent) | Ammonia | Ammonia level | Daily Lab Log | High (Compliance Target) |
| Process (Aeration Tank) | Dissolved Oxygen (DO) | Oxygen concentration in the tank | Hourly SCADA | Critical (Aeration Model) |
| Process (Aeration Tank) | MLSS | Mixed Liquor Suspended Solids | Hourly / Daily | Critical (Biomass state) |
| Process (Aeration Tank) | MLVSS | Mixed Liquor Volatile Suspended Solids | Hourly / Daily | High |
| Process (Aeration Tank) | SVI | Sludge Volume Index | Daily | Medium |
| Process (Aeration Tank) | RAS Flow Rate | Return Activated Sludge Flow | Hourly SCADA | High |
| Process (Aeration Tank) | WAS Flow Rate | Waste Activated Sludge Flow | Hourly SCADA | High |
| Equipment & Operations | Plant Inflow Rate | Total volume of water entering | Hourly SCADA | Critical |
| Equipment & Operations | Blower Status/Speed | Aeration blower Hz/RPM or ON/OFF | Hourly SCADA | Critical (Blower Control) |
| Equipment & Operations | Blower Airflow | Airflow rates (Nm3/hr) | Hourly SCADA | Critical |
| Equipment & Operations | Energy Consumption | Power draw (kW/kWh) for blowers/pumps | Hourly / Daily | High (Cost Optimization) |
| Equipment & Operations | Chemical Dosing Rates | Coagulants, pH adjusters, polymers | Hourly / Daily | Critical (Dosing Model) |
| Events & Maintenance | Maintenance Logs | Breakdowns, pump failures, sensor cals | Event-based | Critical (Anomaly Detection) |
| Events & Maintenance | Process Upsets | Toxic shock, bulking, foaming | Event-based | Critical (Anomaly Detection) |

Requested Format: Excel (.xlsx) or CSV files.